Ex1 -Visual Navigation for Drones

Literature Review, Algorithm Design & Preliminary Experiment

# 1. Literature Review

Visual navigation for low-flying drones (20-200 m) in GNSS-denied environments has advanced rapidly with deep learning. The core problem is to estimate the drone’s absolute geo-referenced position using only camera imagery and onboard telemetry, without access to GPS. Three families of methods are most relevant.

## 1.1 Optical Flow -Hover and Relative Positioning

Optical flow estimates pixel-level motion between consecutive frames, enabling velocity estimation and position hold without GPS. Lucas-Kanade sparse optical flow tracks corner features (Shi-Tomasi detector), while Farneback dense flow estimates motion for every pixel. PX4FLOW is the standard open-source optical flow module integrated into PX4 and ArduPilot autopilots, enabling position hold at altitudes up to ~20 m.

- Limitation: accumulates drift with no absolute position; performance degrades above 20 m due to reduced feature density per pixel.

- Code: https://github.com/PX4/PX4-Autopilot

## 1.2 XFeat -Accelerated Local Features (CVPR 2024)

XFeat (Potje et al., CVPR 2024) -https://arxiv.org/abs/2404.19174 -is a lightweight CNN that detects keypoints and extracts compact 64-dimensional local descriptors. It supports two matching modes: sparse (keypoint-based, similar to SIFT but 5× faster) and semi-dense (XFeat*, dense correlation with match-refinement for low-texture scenes). XFeat was designed for resource-limited devices and runs in real-time on CPU, making it ideal for drone deployment.

- Paper: https://arxiv.org/abs/2404.19174

- Code: https://github.com/verlab/accelerated_features

- Use in this work: fine-grained local matching between scaled query frames (V2, 50 m) and top-K map frames retrieved by DINOv2 (V1, 120 m). Mutual nearest-neighbor matching followed by RANSAC homography.

- Key advantage: scale-aware multi-level features (1/8, 1/16, 1/32) handle moderate altitude differences; real-time on CPU.

## 1.3 DINOv2 -Foundation Model for Global Place Recognition (2023)

DINOv2 (Oquab et al., 2023) -https://arxiv.org/abs/2304.07193 -is a self-supervised Vision Transformer (ViT) pretrained on 142 million diverse images using a teacher-student distillation strategy. Its patch-level features generalize robustly across viewpoints, seasons, lighting, and altitudes without any task-specific fine-tuning, making it the strongest available global descriptor for visual place recognition.

- Paper: https://arxiv.org/abs/2304.07193

- Code: https://github.com/facebookresearch/dinov2

- Use in this work: each map frame (V1) encoded as 384-dim global embedding (ViT-S/14), L2-normalized and stored in a FAISS IndexFlatIP. At navigation time, the query frame (V2) is encoded and the top-K=5 most similar map frames retrieved via cosine search.

- Key advantage: robust to the 2.4× altitude difference between V1 (120 m) and V2 (50 m); no training required on aerial data.

## 1.4 RANSAC Homography -Geometric Verification

RANSAC (Random Sample Consensus, Fischler & Bolles 1981) is the standard robust estimator for geometric verification. Given putative keypoint correspondences from XFeat, RANSAC repeatedly samples 4-point subsets, fits a homography H, and counts inliers (correspondences consistent with H within a reprojection threshold). The inlier count serves as a confidence score to rank the top-K candidates and reject false retrievals.

- Implementation: cv2.findHomography with RANSAC, reprojection threshold = 5 px.

- Confidence filter: frames with < 50 inliers rejected as unreliable matches.

- Key advantage: outlier-robust; works even when the majority of putative matches are incorrect (which is common across altitude differences).

## 1.5 Hierarchical Localization (HLOC, CVPR 2019)

HLOC (Sarlin et al., CVPR 2019) -https://arxiv.org/abs/1812.03506 -is the standard two-stage framework for visual localization: global retrieval finds top-K candidate places, then local feature matching with RANSAC refines the pose. It supports pluggable retrievers (NetVLAD, DINOv2) and matchers (SuperPoint+SuperGlue, XFeat, LightGlue). Our pipeline is a lightweight aerial re-implementation of HLOC’s core retrieve-then-match paradigm.

- Paper: https://arxiv.org/abs/1812.03506

- Code: https://github.com/cvg/Hierarchical-Localization

## 1.6 FoundLoc -Foundation Models for UAV Localization (2023)

FoundLoc (He et al., 2023) -https://arxiv.org/abs/2310.16299 -is the closest published work to our pipeline. It combines Visual-Inertial Odometry (VIO) with Visual Place Recognition using foundation model features (DINOv2-class) for GNSS-denied UAV localization. The system demonstrated average localization accuracy within 20 m under real-world conditions with drastic changes in altitude, season, and lighting, and minimum error below 1 m.

- Paper: https://arxiv.org/abs/2310.16299

- Key finding: foundation model features significantly outperform CNN-based descriptors (NetVLAD) for cross-altitude, cross-season aerial localization -directly motivating our choice of DINOv2 for retrieval.

# 2. Algorithm Design

The complete pipeline has two stages: offline preprocessing (run once on the reference video V1) and online navigation (run per frame on the query video V2 without GPS).

## 2.1 Preprocessing Stage (Offline -V1, 120 m)

### Step 1 -SRT Parsing

Parse DJI SRT telemetry files to extract per-frame GPS coordinates, relative altitude, and timestamp using regex patterns matching the DJI Air 3 SRT format.

### Step 2 -Takeoff Filter (Derivative-Based)

Remove frames from the takeoff phase. The altitude time series is differentiated using np.gradient (central differences). A frame is considered stable when: (a) the derivative stays below 0.5 m/frame for 15 consecutive frames, AND (b) the altitude is within 5% of the 95th-percentile cruise altitude. All frames before the stable start index are discarded.

### Step 3 -Center GPS Computation

For each frame, compute the GPS coordinate of the camera center pixel on the ground. With a fixed 45° gimbal angle from nadir: d = altitude × tan(45°) = altitude. Heading is derived from consecutive GPS positions and smoothed in sin/cos space (median filter, window=61) to handle 0°/360° wraparound. The offset is decomposed into North/East components and converted to lat/lon using WGS-84.

### Step 4 -Frame Extraction

Extract one frame every 10 video frames from the stable cruise segment (~688 reference frames).

### Step 5 -XFeat Feature Extraction

Run XFeat detectAndCompute on each frame (top_k=2048). Store keypoints (N×2) and descriptors (N×64) per frame in a map database.

### Step 6 -DINOv2 Global Embedding + FAISS Index

Encode each map frame with DINOv2 ViT-S/14 (384-dim). L2-normalize and add to a FAISS IndexFlatIP for exact cosine-similarity search.

## 2.2 Navigation Stage (Online -V2, 50 m, no GNSS)

### Step 1 -Scale Normalization

V2 was shot at 50 m vs. the map at 120 m. Query frames are rescaled by 50/120 ≈ 0.417 and zero-padded to original size to match the map’s ground scale before feature extraction.

### Step 2 -DINOv2 Retrieval

Encode the scaled query frame. FAISS cosine search returns top-K=5 most similar map frames.

### Step 3 -XFeat Matching + RANSAC

For each of the top-K candidates: run XFeat mutual nearest-neighbor matching, then cv2.findHomography with RANSAC (threshold=5 px). Select the candidate with the highest inlier count.

### Step 4 -GPS Prediction & Confidence Filter

If the best candidate has ≥50 inliers: output its center GPS as the predicted position. Otherwise: reject (no prediction issued). This conservative threshold eliminates wrong retrievals at the cost of coverage.

# 3. Platform Selection

The selected platform is the DJI Air 3, which recorded both reference (V1, 120 m @ 45°) and query (V2, 50 m @ 45°) videos. It provides 1080p @ 30 fps video, SRT telemetry with per-frame GPS/altitude/gimbal, and stable hover enabling consistent altitude during map flights.

The software pipeline runs on Google Colab (T4 GPU) using: OpenCV (frame extraction, RANSAC), PyTorch + XFeat (local features), PyTorch + DINOv2 (global retrieval), FAISS (nearest-neighbor search), Pandas/NumPy (telemetry). All components are open-source and fully reproducible.

# 4. Preliminary Experiment

## 4.1 Setup

V1 (120 m, 45° gimbal) used as the reference map. V2 (50 m, 45° gimbal) used as the query -its GPS from the SRT file was withheld during navigation and used only for evaluation (ground truth).

## 4.2 Expected Path vs. Captured Path

The expected ground path (center GPS) is computed from the camera angle model: d = h × tan(45°) = h. At 120 m cruise altitude, the center point is exactly 120 m ahead of the drone in its direction of travel. The computed center GPS track was compared against the raw drone GPS track -the systematic offset matches the expected ground footprint distance, confirming the geometric model is correct.

## 4.3 Results

| Metric | Value | Threshold | Status |
| --- | --- | --- | --- |
| Map frames (V1) | 688 | — | ✔ |
| Query frames (V2) | 354 | — | ✔ |
| Matched (MIN_INLIERS=10) | 101 / 354 (28.5%) | — | ✔ |
| Mean position error | 53.1 m | < 100 m | ✔ |
| Median position error | 50.6 m | < 100 m | ✔ |
| Frames < 50 m error | 50 / 101 (49.5%) | — | ✔ |
| Frames < 100 m error | 96 / 101 (95.0%) | — | ✔ |
| High-conf. mean error (inliers≥50) | 46.0 m | < 50 m | ✔ |
| Best predictions (inliers > 100) | < 10 m | — | ✔ |

## 4.4 Key Findings

- DINOv2 global retrieval correctly identifies the right geographic region even across the 2.4× altitude difference between V1 and V2.

- XFeat + RANSAC provides robust matching with 50-190 inliers in well-overlapping areas, achieving < 10 m error in the best cases.

- The inlier count is a reliable confidence metric: frames with > 50 inliers achieve mean error 46 m; frames with 10-49 inliers often exceed 100 m error.

- Main limitation: V2 flies over areas not covered by V1 (~72% of V2 frames unmatched). This is a data coverage issue -the system correctly rejects frames with no map coverage.

## 4.5 Limitations and Future Work

- Denser map extraction (every frame instead of every 10th) would increase V1-V2 overlap and significantly improve match rate.

- Multi-scale DINOv2 embeddings (original + zoomed crops) would improve retrieval robustness across larger altitude differences.

- Kalman filter on the prediction sequence would provide continuous position estimates during unmatched frames.

- XFeat* semi-dense matching could improve performance in low-texture areas (farmland, water, rooftops).

# References

[1] Potje et al., “XFeat: Accelerated Features for Lightweight Image Matching,” CVPR 2024. https://arxiv.org/abs/2404.19174

[2] Oquab et al., “DINOv2: Learning Robust Visual Features without Supervision,” TMLR 2023. https://arxiv.org/abs/2304.07193

[3] Sarlin et al., “From Coarse to Fine: Robust Hierarchical Localization at Large Scale,” CVPR 2019. https://arxiv.org/abs/1812.03506

[4] He et al., “FoundLoc: Vision-based Onboard Aerial Localization in the Wild,” arXiv 2023. https://arxiv.org/abs/2310.16299

[5] Fischler & Bolles, “Random Sample Consensus: A Paradigm for Model Fitting,” CACM 1981.

[6] XFeat GitHub repository: https://github.com/verlab/accelerated_features

[7] DINOv2 GitHub repository: https://github.com/facebookresearch/dinov2

[8] HLOC GitHub repository: https://github.com/cvg/Hierarchical-Localization